lcd-i2c-driver

STM32 HAL LCD Driver via PCF8574 I2C Expander

A lightweight STM32 HAL-based C driver for controlling 16x2 LCD displays via the PCF8574 I2C I/O expander. Operates in 4-bit mode and provides a clean, easy-to-use API for display initialization, text rendering, and animation.

## Features

- 4-bit LCD communication over I2C using the PCF8574 expander

- Display initialization, clear, and cursor positioning

- Write functions: characters, strings, integers, floats, and hex values

- Animated bounce text effect (left-right scroll)

- Backlight always enabled via PCF8574 BL pin

## Hardware Requirements

| Component | Details |
| --- | --- |
| MCU | STM32 (any HAL-compatible with I2C) |
| LCD | 16x2 HD44780-compatible |
| I2C Expander | PCF8574 |
| Interface | I2C — default: hi2c1 |

### PCF8574 Pin Mapping

| PCF8574 Bit | LCD Pin | Description |
| --- | --- | --- |
| 0x01 | RS | Register Select |
| 0x02 | RW | Read/Write (always write) |
| 0x04 | EN | Enable |
| 0x08 | BL | Backlight Control |

## File Structure

lcd-i2c-driver/

├── lcd_i2c.c       # Driver implementation

├── lcd_i2c.h       # Header: declarations & config

└── README.md

## Configuration

In lcd_i2c.h, set your PCF8574 I2C address:

#define LCD_I2C_ADDR   0x4E   // 0x27 << 1 for PCF8574

The driver references hi2c1 via extern. Update lcd_i2c.c if using a different I2C peripheral.

## API Reference

### Initialization & Control

| Function | Description |
| --- | --- |
| LCD_Init() | Initializes LCD in 4-bit mode. Call once after boot. |
| LCD_Clear() | Clears the display and resets cursor to home. |
| LCD_SetCursor(row, col) | Moves cursor to row (0–1) and col (0–15). |

### Write Functions

| Function | Description |
| --- | --- |
| LCD_WriteChar(c) | Writes a single character at cursor position. |
| LCD_WriteString(str) | Writes a null-terminated string. |
| LCD_WriteInt(num) | Writes a signed integer. |
| LCD_WriteFloat(num, precision) | Writes a float with specified decimal places. |
| LCD_WriteHex(num) | Writes a 32-bit uint in 0xHEX format. |

### Animation

| Function | Description |
| --- | --- |
| LCD_BounceText(str, row, delay_ms) | Slides text left and right across a row. Strings > 16 chars are truncated. |

## Usage Example

#include "lcd_i2c.h"

int main(void) {

HAL_Init();

SystemClock_Config();

MX_I2C1_Init();

LCD_Init();

LCD_SetCursor(0, 0);

LCD_WriteString("Hello, World!");

LCD_SetCursor(1, 0);

LCD_WriteFloat(3.14159, 2);  // Displays: 3.14

while (1) {

LCD_BounceText("STM32 LCD", 0, 200);

}

}

## Notes

- RW line is always pulled low — the driver is write-only.

- HAL_Delay() is used for timing; ensure SysTick is initialized before LCD_Init().

- Backlight is always on (LCD_BL is ORed into every PCF8574 write).

- Strings longer than 16 characters in LCD_BounceText() are silently truncated to 16.

Author: Aryan  |  February 2026  |  MIT License